Camera Subsystem

Technical User Guide

OV7670 Camera Module → DE10-Lite FPGA

Covers: camera_subsystem.vhd • i2c_top.vhd • camera_interface.vhd • asyn_fifo.vhd

1  Overview	1

1.1  What the subsystem does	1

1.2  Internal components	1

1.3  Data flow	1

2  Port Reference — camera_subsystem	1

2.1  Clock and reset	1

2.2  OV7670 physical interface	1

2.3  FIFO read interface (to sdram_subsystem)	1

2.4  Status outputs	1

3  How i2c_top Works — SCCB Register Initialisation	1

3.1  Startup sequence	1

3.2  SCCB transaction format	1

3.3  Register table — what gets written	1

3.4  SCCB electrical requirements	1

3.5  Modifying register settings	1

4  How camera_interface Works — Pixel Capture FSM	1

4.1  OV7670 pixel timing	1

4.2  FSM states	1

4.3  Pixel assembly — step by step	1

4.4  Line boundary re-alignment	1

4.5  Full FIFO behaviour	1

4.6  frame_done pulse	1

5  How asyn_fifo Works — Clock Domain Crossing	1

5.1  The problem: metastability	1

5.2  Why not just use a synchroniser for data?	1

5.3  Gray-code pointer synchronisation	1

5.4  Full and empty detection	1

5.5  FIFO configuration in this project	1

5.6  Reading from the FIFO (sdram_subsystem perspective)	1

6  Clock Domain Crossing Summary	1

7  End-to-End Timing from Power-On	1

8  Troubleshooting	1

8.1  i2c_done never asserts (LEDR[2] stays off)	1

8.2  i2c_done asserts but no image appears	1

8.3  Image has wrong colours	1

8.4  Image rolls or has horizontal tears	1

8.5  Changing the register table	1

9  Design Decisions and Rationale	1

9.1  Why i2c_done gates the capture FSM	1

9.2  Why frame_done is 2 cycles wide	1

9.3  Why the FIFO depth is 512 (not larger)	1

9.4  Why PCLK is used for capture, not clk_24m	1

1  Overview

The camera subsystem is the first stage of the real-time video streaming pipeline on the DE10-Lite FPGA. It takes raw electrical signals from an OV7670 camera module and produces a stream of 16-bit RGB565 pixels that the SDRAM subsystem can store in the frame buffer.

The subsystem is entirely automatic: it configures the camera over SCCB on startup, then captures pixels continuously without any software intervention. It is implemented in pure VHDL and contains no soft-core processor.

1.1  What the subsystem does

- Programs the OV7670 camera with all required settings (RGB565 format, 640×480 resolution) by writing 30 registers over SCCB at startup.

- Captures pixel bytes from the 8-bit parallel data bus on every PCLK edge when HREF is high.

- Assembles pairs of bytes into one 16-bit RGB565 word and writes it into a FIFO.

- Bridges the clock domain crossing from the camera’s 24 MHz PCLK into the SDRAM’s 165 MHz clock using a gray-code asynchronous FIFO.

- Signals frame_done to the SDRAM subsystem at the end of each frame so it can swap the double-buffer.

1.2  Internal components

| Component | File | Role |
| --- | --- | --- |
| u_i2c | i2c_top.vhd | SCCB master — programs all OV7670 registers on startup, controls RESET_N and PWDN |
| u_cam | camera_interface.vhd | Pixel capture FSM — assembles D[7:0] byte pairs into RGB565 words |
| u_fifo | asyn_fifo.vhd | Asynchronous FIFO — bridges PCLK (24 MHz write) to clk_165m (165 MHz read) |

1.3  Data flow

OV7670 hardware → u_i2c configures camera via SCCB → OV7670 outputs PCLK + HREF + VSYNC + D[7:0] → u_cam assembles RGB565 pixels → u_fifo crosses clock domain → sdram_subsystem reads pixels at 165 MHz

2  Port Reference — camera_subsystem

The subsystem entity exposes only the signals that cross its boundary. Internal wiring (camera_interface to FIFO, i2c_done to capture gate) is hidden inside the wrapper.

2.1  Clock and reset

| Signal | Direction | Description |
| --- | --- | --- |
| clk_24m | in | 24 MHz from pll_main c0. Drives i2c_top and the XCLK output to the camera. Must be stable before rst_n deasserts. |
| clk_165m | in | 165 MHz from pll_main c2. Drives the read side of the asyn_fifo. Must be stable before rst_n deasserts. |
| rst_n | in | Active-low global reset. Deasserts only after all PLLs are locked (sys_ctrl). Holds i2c_top and camera_interface in reset. |

2.2  OV7670 physical interface

| Signal | Direction | Description |
| --- | --- | --- |
| cam_pclk | in | Pixel clock generated by the OV7670 after XCLK is applied (~24 MHz). Clocks the capture FSM and the FIFO write side. |
| cam_vsync | in | Frame sync from OV7670. Active high = vertical blanking. camera_interface uses rising and falling edges to delimit frames. |
| cam_href | in | Line valid from OV7670. Active high = pixel data on D[7:0] is valid. Falling edge resets byte alignment at end of each line. |
| cam_d | in | Parallel pixel data bus, 8 bits. Two consecutive bytes (on successive PCLK edges while HREF=1) form one RGB565 pixel. |
| cam_xclk | out | Master clock driven to the OV7670. Tied directly to clk_24m (24 MHz). The camera uses this to generate its own PCLK. |
| cam_sioc | out | SCCB clock (open-drain). Requires 4.7kΩ external pull-up to 3.3V. Driven by i2c_top during register initialisation. |
| cam_siod | inout | SCCB data (open-drain, bidirectional). Requires 4.7kΩ external pull-up to 3.3V. |
| cam_rst_n | out | Hardware reset to OV7670 (active low). Held low for 5 ms by i2c_top, then released before SCCB begins. |
| cam_pwdn | out | Power-down control. Permanently driven low (camera always powered). Connect to OV7670 PWDN pin. |

2.3  FIFO read interface (to sdram_subsystem)

| Signal | Direction | Description |
| --- | --- | --- |
| fifo_rd_en | in | Read enable from sdram_subsystem. Assert for one clk_165m cycle to pop one pixel from the FIFO. |
| fifo_rd_data | out | 16-bit RGB565 pixel output. Valid one clk_165m cycle after fifo_rd_en is asserted (registered output). |
| fifo_empty | out | High when the FIFO contains no pixels. sdram_subsystem must not assert fifo_rd_en when this is high. |

2.4  Status outputs

| Signal | Direction | Description |
| --- | --- | --- |
| frame_done | out | Pulse in the PCLK domain, 2 PCLK cycles wide. Asserts when VSYNC rises inside S_CAPTURE (end of active frame). sdram_subsystem re-synchronises this into clk_165m using a 2-FF synchroniser. |
| i2c_done | out | High once i2c_top has written all OV7670 registers. Remains high permanently. Useful as a debug LED (LEDR[2]). |

3  How i2c_top Works — SCCB Register Initialisation

i2c_top is an SCCB (Serial Camera Control Bus) master. SCCB is electrically identical to I²C but uses a simpler 3-phase write-only protocol. i2c_top runs entirely on clk_24m and produces SIOC at 100 kHz.

3.1  Startup sequence

After rst_n deasserts, i2c_top proceeds through a fixed startup sequence automatically:

| Phase | Duration | What happens |
| --- | --- | --- |
| ST_RESET_HOLD | 5 ms (120,000 cycles @ 24 MHz) | cam_rst_n driven low. OV7670 is held in hardware reset. All SCCB lines idle high. |
| ST_RESET_RELEASE | 5 ms (120,000 cycles @ 24 MHz) | cam_rst_n released high. Camera boots its internal state. SCCB not yet started. |
| ST_IDLE → ST_START | one SCCB tick | First register (COM7 = 0x80, soft reset) is queued for transmission. |
| ST_SEND_BYTE ×3 | ~240 µs per register | Device address (0x42), register address, data byte sent MSB first at 100 kHz. |
| ST_SOFT_RST_WAIT | 1 ms (24,000 cycles @ 24 MHz) | Mandatory pause after soft reset. OV7670 requires 1 ms before accepting further writes. |
| Remaining registers | ~30 registers × 240 µs ≈ 7 ms | All 29 remaining register values transmitted sequentially. |
| ST_DONE | permanent | done_int = 1. SIOC and SIOD released idle high. camera_interface begins capturing. |

Total time from rst_n deassert to i2c_done: approximately 5 + 5 + 0.24 + 1 + 7 = ~18 ms. The camera will produce its first valid pixel frame roughly 33 ms after reset.

3.2  SCCB transaction format

Every register write produces one SCCB transaction consisting of three bytes, each 8 bits with a 9th don’t-care ACK bit:

START | Device address (0x42) | ACK | Register address | ACK | Data byte | ACK | STOP

Example: write COM7 = 0x04  (RGB mode)
START | 0x42 | ACK | 0x12 | ACK | 0x04 | ACK | STOP

Unlike I²C, SCCB does not require the slave to assert the ACK bit. The master releases SDA during the 9th clock pulse and ignores whatever the slave drives. The i2c_top ACK state simply gives one SCL pulse with SDA released (pulled high externally).

3.3  Register table — what gets written

| Register | Addr | Value | Effect |
| --- | --- | --- | --- |
| COM7 | 0x12 | 0x80 | [0] Soft reset — resets all registers to factory defaults. 1 ms delay follows automatically. |
| CLKRC | 0x11 | 0x01 | Pre-scalar /2: internal clock = XCLK/2 = 12 MHz |
| DBLV | 0x6B | 0x4A | Bypass internal PLL. Use external clock directly. |
| COM7 | 0x12 | 0x04 | RGB output mode selected. No QVGA down-scaling. |
| COM15 | 0x40 | 0xD0 | RGB565 format. Full output range 0x00–0xFF. |
| RGB444 | 0x8C | 0x00 | Disable RGB444 alternate format. |
| COM14 | 0x3E | 0x00 | No PCLK division, no manual scaling. |
| SCALING_* | 0x70–0x73 | see code | Scaling registers for 640×480 active window. |
| HSTART/HSTOP | 0x17–0x18 | 0x13/0x01 | Horizontal window: 640 active pixels. |
| VSTART/VSTOP | 0x19–0x1A | 0x02/0x7A | Vertical window: 480 active lines. |
| COM8 | 0x13 | 0xE0 | Disable AGC, AWB, AEC. Manual gain/exposure for stable colour. |
| COM10 | 0x15 | 0x00 | VSYNC active high, HREF not inverted. Must match camera_interface expectations. |
| MTX1–MTXS | 0x4F–0x58 | various | Colour correction matrix for accurate RGB output. |
| Sentinel | 0xFF | 0xFF | End marker. i2c_top detects this and asserts done. |

COM10 (0x15 = 0x00) sets VSYNC active high. If your OV7670 module produces VSYNC active low, change this register to 0x02 AND invert the vsync logic in camera_interface.vhd S_WAIT_VSYNC_HIGH and S_CAPTURE states.

3.4  SCCB electrical requirements

- SIOC and SIOD are open-drain outputs. The VHDL drives ‘0’ or releases to ‘Z’ (never drives ‘1’).

- Both lines require 4.7 kΩ pull-up resistors to 3.3V placed close to the camera module.

- Without pull-ups, both lines float undefined and no registers will be written.

- The level shifter (TXS0108E) must be in place between the FPGA (3.3V) and the OV7670 (1.8V I/O).

3.5  Modifying register settings

To change camera behaviour (colour, gain, window size), edit the REG_TABLE constant in i2c_top.vhd. Add, remove, or change (address, data) pairs. The sentinel (0xFF, 0xFF) must remain last. After editing, recompile and reprogram the FPGA.

-- Example: enable Auto White Balance (change COM8)
-- Find this line in REG_TABLE:
(x"13", x"E0"),  -- COM8: disable AGC/AWB/AEC
-- Change to:
(x"13", x"E7"),  -- COM8: enable AGC + AWB + AEC

4  How camera_interface Works — Pixel Capture FSM

camera_interface runs entirely on the OV7670 PCLK (~24 MHz). It monitors the VSYNC and HREF control signals and assembles the 8-bit D[7:0] byte pairs into 16-bit RGB565 words that it writes into the asyn_fifo.

4.1  OV7670 pixel timing

The OV7670 outputs pixels in a strict sequence. Understanding this timing is essential to understanding how camera_interface works:

VSYNC: ______|"""""""""|___________|"""""""""|___  (high = blanking)
             ^frame 1 ^ ^frame 1   ^ ^frame 2^
              blanking    active        blanking

HREF:  ___|""""""""""|_|""""""""""|_|""""""""""| (high = line active)
           line 0      line 1      line 2

PCLK:  clk clk clk clk clk clk clk clk clk clk

D[7:0]: X  [B0H][B0L] [B1H][B1L] [B2H][B2L]  X
             pixel 0    pixel 1    pixel 2

B0H = first byte of pixel 0  : [R4:R0][G5:G3]
B0L = second byte of pixel 0 : [G2:G0][B4:B0]
Assembled: pixel[15:0] = B0H & B0L = RGB565

4.2  FSM states

| State | Behaviour |
| --- | --- |
| S_WAIT_CONFIG | Initial state. Waits here until i2c_done = 1. This prevents the FSM from seeing garbage VSYNC/HREF signals while the camera is being configured. byte_sel is held 0. |
| S_WAIT_VSYNC_HIGH | Waits for VSYNC to rise (start of vertical blanking). This marks the boundary between frames. byte_sel reset to 0 to clear any partial pixel. |
| S_WAIT_VSYNC_LOW | Waits for VSYNC to fall (active image area begins). If HREF is already high when VSYNC falls, latches the first byte immediately. |
| S_CAPTURE | Main capture state. On each PCLK edge: if HREF=1, toggles byte_sel to assemble byte pairs into pixels. If VSYNC rises, transitions to S_FRAME_DONE. If HREF falls, resets byte_sel to re-align on next line. |
| S_FRAME_DONE | Holds frame_done high for 2 PCLK cycles for safe CDC crossing, then returns to S_WAIT_VSYNC_HIGH for the next frame. |

4.3  Pixel assembly — step by step

In S_CAPTURE, for each pair of PCLK edges where HREF=1:

- byte_sel = 0: cam_data latched into d_latch. byte_sel set to 1.

- byte_sel = 1: pixel_out assembled as d_latch & cam_data (16-bit RGB565 word). If fifo_full=0, wr_en_int asserted for one cycle to write the pixel into the FIFO. byte_sel reset to 0.

-- Inside S_CAPTURE when href = 1:
if byte_sel = 0 then
    d_latch  <= cam_data;       -- latch high byte [R4:R0][G5:G3]
    byte_sel <= 1;
else
    pixel_out <= d_latch & cam_data;  -- assemble RGB565
    byte_sel  <= 0;
    if fifo_full = 0 then
        wr_en_int <= 1;         -- write to FIFO
    end if;
    -- if fifo_full=1: pixel is silently dropped (camera cannot pause)
end if;

4.4  Line boundary re-alignment

When HREF falls (end of a scan line), byte_sel is forced to 0 regardless of its current value. This prevents a half-assembled pixel from contaminating the next line if, for any reason, an odd number of bytes was seen on the previous line.

elsif href = 0 and href_prev = 1 then
    -- Falling edge of HREF: end of scan line
    byte_sel <= 0;   -- force re-alignment for next line
end if;

4.5  Full FIFO behaviour

The camera cannot be paused — PCLK, HREF and pixel data continue regardless. When the FIFO is full, pixels are silently dropped. This is acceptable because:

- The FIFO drain rate (165 MHz read side) is 6.875× faster than the fill rate (24 MHz PCLK).

- The 512-entry FIFO depth provides sufficient headroom for CDC latency and any SDRAM refresh pauses.

- Under normal operation, the FIFO never actually fills up.

If LEDR[3] (cam_fifo_empty) is always lit and no image appears, the FIFO is never receiving data. Check: i2c_done is asserted (LEDR[2]), cam_pclk is connected and switching, HREF and VSYNC are reaching the FPGA through the level shifter.

4.6  frame_done pulse

At the end of each active frame (VSYNC rising edge detected in S_CAPTURE), frame_done is asserted for exactly 2 PCLK cycles. Two cycles is the minimum safe width for the 2-FF synchroniser in sdram_subsystem to reliably capture the pulse when crossing from 24 MHz to 165 MHz.

-- In S_FRAME_DONE:
if frame_cnt > 0 then
    frame_done_r <= 1;         -- hold pulse high
    frame_cnt    <= frame_cnt - 1;
else
    frame_done_r <= 0;
    state        <= S_WAIT_VSYNC_HIGH;  -- ready for next frame
end if;

5  How asyn_fifo Works — Clock Domain Crossing

The asyn_fifo bridges the write clock domain (cam_pclk at ~24 MHz, from the camera) and the read clock domain (clk_165m at 165 MHz, from the PLL). These two clocks are completely independent and have no fixed phase relationship — they cannot share flip-flops without metastability protection.

5.1  The problem: metastability

When a signal crosses from one clock domain to another, the receiving flip-flop may sample the signal exactly as it is changing. This puts the flip-flop into a metastable state where its output is neither 0 nor 1 for an indeterminate time. If metastability propagates into logic, the FPGA produces unpredictable results.

The solution is to never use a raw signal directly across clock domains. Instead, use a synchroniser: two flip-flops in series clocked by the destination clock. The first FF may go metastable, but it has almost the entire clock period to resolve before the second FF samples it. The probability of the second FF also going metastable is negligibly small.

5.2  Why not just use a synchroniser for data?

A 2-FF synchroniser works for single-bit control signals. It does not work for multi-bit data because each bit may resolve at a different time, producing a garbled word. The asyn_fifo solves this by:

- Storing data in a shared dual-port RAM (which does not cross clock domains — it is accessed independently from each side).

- Only crossing single-bit gray-code pointer values between domains. Gray code changes only one bit at a time, so even if a synchroniser samples it mid-transition, the result is either the old or new pointer value — never a garbled intermediate.

5.3  Gray-code pointer synchronisation

The FIFO maintains binary write and read pointers. Before crossing clock domains, each pointer is converted to Gray code:

-- Binary to Gray conversion (bin2gray function):
g(MSB) := b(MSB);
for i in MSB-1 downto 0 loop
    g(i) := b(i+1) xor b(i);
end loop;

-- Example: binary 0111 (7) -> gray 0100
--          binary 1000 (8) -> gray 1100  (only 1 bit changed)

Each gray-code pointer is then passed through a 2-FF synchroniser into the opposite domain:

- Write domain: rd_gray_sync2 is the synchronised version of rd_ptr_gray. Used to detect full condition.

- Read domain: wr_gray_sync2 is the synchronised version of wr_ptr_gray. Used to detect empty condition.

5.4  Full and empty detection

-- EMPTY: read pointer has caught up with write pointer
empty_int <= 1 when rd_ptr_gray = wr_gray_sync2 else 0;

-- FULL: write pointer has wrapped around and is one step behind read pointer
-- In gray code, full is detected when:
--   MSB differs (write has wrapped) AND
--   MSB-1 differs (wrap confirmation) AND
--   remaining bits are equal (same position within the block)
full_int <= 1 when
    wr_ptr_gray(MSB)   /= rd_gray_sync2(MSB)   and
    wr_ptr_gray(MSB-1) /= rd_gray_sync2(MSB-1) and
    wr_ptr_gray(MSB-2 downto 0) = rd_gray_sync2(MSB-2 downto 0)
else 0;

5.5  FIFO configuration in this project

| Parameter | Value | Reason |
| --- | --- | --- |
| DATA_WIDTH | 16 bits | One RGB565 pixel per entry |
| DEPTH_LOG2 | 9 | 2⁹ = 512 entries = 512 pixels |
| Depth in bytes | 1024 bytes | Covers ~0.8 scan lines of pixel data |
| Write clock | cam_pclk (~24 MHz) | Camera-generated pixel clock |
| Read clock | clk_165m (165 MHz) | SDRAM controller clock from PLL c2 |
| Drain ratio | 6.875× faster than fill | SDRAM can drain the FIFO much faster than camera fills it |
| Read output | Registered (1-cycle latency) | rd_data valid one clk_165m cycle after rd_en asserted |

5.6  Reading from the FIFO (sdram_subsystem perspective)

- Check fifo_empty. If high, no data available — do not assert fifo_rd_en.

- Assert fifo_rd_en for one clk_165m cycle.

- Wait one clk_165m cycle.

- Read fifo_rd_data. The 16-bit RGB565 pixel is now valid.

- Repeat from step 1.

The registered read output means fifo_rd_data is valid ONE cycle after fifo_rd_en is asserted. The sdram_interface accounts for this with its ST_WAIT_CAM_DATA state.

6  Clock Domain Crossing Summary

The camera subsystem manages two clock domain crossings. Both are handled safely by design.

| Signal | From domain | To domain | Mechanism |
| --- | --- | --- | --- |
| pixel data (16-bit) | cam_pclk ~24 MHz | clk_165m 165 MHz | asyn_fifo with gray-code pointers. Data never crosses directly — stored in shared RAM, only pointers cross via 2-FF synchronisers. |
| frame_done (1-bit) | cam_pclk ~24 MHz | clk_165m 165 MHz | 2-PCLK-wide pulse. Re-synchronised inside sdram_subsystem using a 2-FF synchroniser. Pulse width (2 × 41 ns = 82 ns) is safely longer than 2 × 6 ns = 12 ns minimum for capture. |
| i2c_done (1-bit) | clk_24m 24 MHz | cam_pclk ~24 MHz | Both clocks are nominally 24 MHz from the same PLL. Treated as quasi-synchronous. Static signal (never falls) so safe with or without synchroniser. |

The Quartus QSF file contains set_false_path constraints for the cam_pclk → clk_165m crossing. This tells TimeQuest that these paths are safe by design (gray-code CDC). Do not remove these constraints.

7  End-to-End Timing from Power-On

| Time | Event | Detail |
| --- | --- | --- |
| t = 0 | rst_n deasserts | PLL locked. Global reset released. cam_rst_n = 0 immediately. |
| t ≈ 1 ms | SDRAM init begins | sdram_controller starts its 200 µs power-up wait. Parallel to camera startup. |
| t ≈ 5 ms | cam_rst_n released | i2c_top ST_RESET_HOLD completes (5 ms). cam_rst_n goes high. |
| t ≈ 10 ms | SCCB starts | i2c_top ST_RESET_RELEASE completes (5 ms). First SCCB transaction begins. |
| t ≈ 10.24 ms | Soft reset written | COM7=0x80 transmitted. i2c_top enters ST_SOFT_RST_WAIT (1 ms mandatory). |
| t ≈ 11.24 ms | Remaining regs | 29 registers written at ~240 µs each. |
| t ≈ 18 ms | i2c_done = 1 | All registers written. camera_interface exits S_WAIT_CONFIG. |
| t ≈ 18 ms | Camera outputs valid | OV7670 PCLK running. VSYNC/HREF stable. First valid frame begins. |
| t ≈ 51 ms | First frame done | Frame 0 captured (1/30 s = 33 ms after camera starts). frame_done pulse. |
| t ≈ 51 ms+ | Live streaming | Continuous double-buffered capture and display at 30 fps. |

8  Troubleshooting

8.1  i2c_done never asserts (LEDR[2] stays off)

- Check that SIOC and SIOD have 4.7 kΩ pull-up resistors to 3.3V on the 3.3V side of the level shifter.

- Check that the TXS0108E VA pin is connected to DOVDD (1.8V) and VB to 3.3V.

- Verify clk_24m is reaching i2c_top: the XCLK output (cam_xclk) should be toggling at 24 MHz.

- Check rst_n is deasserted (LEDR[1] lit). i2c_top stays in ST_RESET_HOLD while rst_n=0.

8.2  i2c_done asserts but no image appears

- camera_interface is waiting for i2c_done but also needs a valid VSYNC sequence. Check VSYNC is reaching GPIO_0[10].

- Check cam_pclk is connected to GPIO_0[8]. Without PCLK, the capture FSM never advances.

- Check the level shifter is correctly powered and all signal lines are passing through it.

- Verify FIFO is not always full: LEDR[3] (cam_fifo_empty) should flicker, not stay permanently lit.

8.3  Image has wrong colours

- Confirm COM7 (0x12) second write is 0x04 (RGB mode). If 0x00 it defaults to YUV output.

- Confirm COM15 (0x40) is 0xD0 for RGB565. If 0xC0 it selects RGB555 instead.

- Check the colour matrix registers (MTX1–MTXS) are all being written correctly. A SCCB failure mid-sequence leaves some registers at default values.

8.4  Image rolls or has horizontal tears

- Rolling: VSYNC polarity mismatch. COM10 (0x15 = 0x00) sets VSYNC active high. If your module has active-low VSYNC, change COM10 to 0x02.

- Horizontal tearing: rd_ptr being reset on bank swap in sdram_interface. Ensure you are using the corrected sdram_interface.vhd where rd_ptr is NOT reset on frame_done.

- Colour stripes: byte_sel misalignment. This is handled by the href falling-edge reset in camera_interface.vhd. If you modified the code, check this logic is intact.

8.5  Changing the register table

To modify camera settings, edit the REG_TABLE constant in i2c_top.vhd. Rules:

- The first entry must be (0x12, 0x80) — the soft reset. Do not move it.

- The last entry must be (0xFF, 0xFF) — the sentinel. Do not remove it.

- Add new entries between the soft reset and the sentinel.

- After any change, recompile and reprogram the FPGA. The camera reads registers only at startup.

- i2c_done will assert after the last non-sentinel register is transmitted.

9  Design Decisions and Rationale

9.1  Why i2c_done gates the capture FSM

The OV7670 outputs PCLK, VSYNC, and HREF as soon as XCLK is applied — even before any registers are written. In its default power-on state, the camera outputs YUV data, not RGB565. If camera_interface started capturing immediately after reset, the first frames would contain garbage data in the wrong format.

By gating the FSM on i2c_done, the capture only begins after all registers are confirmed written. This guarantees that the first pixel captured is a valid RGB565 pixel in the correct 640×480 format.

9.2  Why frame_done is 2 cycles wide

A 1-cycle pulse at 24 MHz lasts 41 ns. The 165 MHz domain has a clock period of 6 ns. In the worst case, the 2-FF synchroniser in sdram_subsystem samples the pulse on the second FF just after it deasserts, missing it entirely.

A 2-cycle pulse lasts 82 ns. Even with worst-case metastability resolution time, the 165 MHz domain is guaranteed to see at least one rising edge during the pulse. Two cycles is the minimum safe width.

9.3  Why the FIFO depth is 512 (not larger)

512 entries = 512 pixels = approximately 0.8 scan lines of data. The 165 MHz read side drains the FIFO 6.875× faster than the 24 MHz write side fills it. Even during an SDRAM auto-refresh pause (worst case ~66 ns = 11 cycles at 165 MHz), the FIFO accumulates only a handful of pixels before the drain resumes. 512 entries provides ample margin.

Making the FIFO larger would waste M9K block RAM resources. Making it smaller would risk overflow during SDRAM refresh stalls. 512 is the right balance for this design.

9.4  Why PCLK is used for capture, not clk_24m

clk_24m (from the PLL) and cam_pclk (from the OV7670) are both nominally 24 MHz, but they are not the same clock. The OV7670 generates PCLK internally from its own oscillator. The phase relationship between clk_24m and cam_pclk is unknown and changes with temperature.

If camera_interface were clocked by clk_24m, it would sample HREF and D[7:0] at arbitrary phase offsets relative to when the camera actually presents valid data. Using cam_pclk guarantees that data is sampled exactly when the camera says it is valid (on the PCLK rising edge). This is the only correct approach.