OV7670 → DE10-Lite FPGA

Real-Time VGA Streaming

User Guide

640 × 480 @ 30 fps  •  RGB565  •  SDRAM double-buffer  •  VHDL

Target hardware: Intel MAX10 • DE10-Lite (10M50DAF484C7G)

Tool: Quartus Prime 18.1 Lite Edition

# 1  Project Overview

This project implements a live real-time video streaming pipeline on the Intel MAX10 FPGA (DE10-Lite development board). Raw pixel data is captured from an OV7670 camera module, buffered through the 64 MB onboard SDRAM, and displayed on a VGA monitor at 640 × 480 resolution at 30 fps.

The entire design is written in VHDL and targets Quartus Prime 18.1 Lite Edition. No soft-core processor (Nios II) is used — the pipeline is a fully hardware state-machine implementation across three independent clock domains.

## 1.1  Key features

- 640 × 480 pixels, 30 fps live camera feed

- RGB565 colour format (16-bit per pixel, 65,536 colours)

- Three independent clock domains: 24 MHz (camera), 165 MHz (SDRAM), 25 MHz (VGA)

- Asynchronous FIFOs with gray-code CDC at every clock boundary

- Double-buffered SDRAM frame buffer — eliminates tearing

- SCCB (I²C) automatic camera initialisation on startup

- VUnit testbenches for every module (78 tests total)

## 1.2  System architecture

Data flows in one direction through five subsystems:

| Camera subsystem | → | SDRAM subsystem | → | VGA subsystem | → | Monitor output |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i2c_top camera_interface asyn_fifo |  | sdram_interface sdram_controller (IS42S16320F) |  | asyn_fifo vga_core vga_interface |  | VGA 640×480 4-bit DAC 60 Hz |  |  |

# 2  Hardware Requirements

## 2.1  Required components

| Component | Quantity | Notes |
| --- | --- | --- |
| DE10-Lite FPGA board | 1 | Intel MAX10 10M50DAF484C7G, includes 64 MB SDRAM and VGA connector |
| OV7670 camera module | 1 | With FIFO (AL422B) or without FIFO — project drives pixel capture directly |
| TXS0108E level shifter | 2 | 1.8V ↔ 3.3V, 8-channel bidirectional (Texas Instruments) |
| AMS1117-2.8 LDO | 1 | 2.8V output — supplies OV7670 AVDD and DVDD |
| AMS1117-1.8 LDO | 1 | 1.8V output — supplies OV7670 DOVDD (I/O rail) |
| 4.7 kΩ resistors | 2 | SCCB pull-ups for SIOC and SIOD lines |
| 100 nF ceramic caps | 5 | Decoupling: one per supply pin on OV7670, one on 3.3V header pin |
| 10 µF electrolytic | 3 | Bulk decoupling on AVDD, DVDD, DOVDD rails |
| VGA cable + monitor | 1 | Standard 15-pin D-sub VGA, monitor supporting 640×480 @ 60 Hz |
| Jumper wires | 20+ | Female-to-female for breadboard or connector wiring |

## 2.2  Software tools

- Quartus Prime 18.1 Lite Edition (free, from Intel FPGA download centre)

- USB Blaster driver (included with Quartus, required for DE10-Lite programming)

- ModelSim-Intel FPGA Starter Edition (optional, for simulation)

- GHDL + VUnit (optional, for running the provided testbenches)

# 3  Project File Structure

The project contains 15 VHDL source files organised into four layers. The table below shows the complete file list and the role of each file.

| File | Layer | Description |
| --- | --- | --- |
| stream_top.vhd | Top level | Pure wiring top entity — no logic, just port maps |
| stream_top.qsf | Constraints | All Quartus pin assignments and timing constraints |
| pll_main.vhd | Clock | Single ALTPLL: 24 MHz (camera) + 25 MHz (VGA) + 165 MHz ×0° + 165 MHz −90° (SDRAM) |
| sys_ctrl.vhd | Clock | Reset synchroniser, PLL-lock gating, KEY[0] debounce |
| debounce_explicit.vhd | Utility | Counter-based button debouncer, 20 ms window |
| camera_subsystem.vhd | Camera | Wrapper: i2c_top + camera_interface + asyn_fifo |
| i2c_top.vhd | Camera | SCCB master — writes all OV7670 registers on startup |
| camera_interface.vhd | Camera | Pixel capture FSM — assembles RGB565 from D[7:0] |
| asyn_fifo.vhd | Utility | Gray-code async FIFO — used twice for clock domain crossing |
| sdram_subsystem.vhd | SDRAM | Wrapper: sdram_interface + sdram_controller + 2-FF sync |
| sdram_interface.vhd | SDRAM | Frame-buffer arbitrator, double-buffer bank swap |
| sdram_controller.vhd | SDRAM | IS42S16320F command FSM — init, refresh, read, write |
| vga_subsystem.vhd | VGA | Wrapper: asyn_fifo + vga_core + vga_interface |
| vga_core.vhd | VGA | 640×480 @ 60 Hz VESA timing generator |
| vga_interface.vhd | VGA | Reads FIFO, maps RGB565 to DE10-Lite 4-bit DAC outputs |

# 4  PLL Configuration (Quartus ALTPLL)

The PLL is the only IP core in the project. It must be generated using the Quartus IP Catalog wizard before compiling. The pll_main.vhd file already contains the wrapper; you only need to generate the underlying altpll_core component.

## 4.1  Generating the PLL

- Open Quartus Prime → IP Catalog (menu: Tools → IP Catalog).

- Search for “ALTPLL” and double-click to open the wizard.

- Set the output file name to altpll_core.vhd inside your project folder.

- On page 1, set the input clock frequency to 50.000 MHz.

- On the Output Clocks tab, configure four outputs as shown in the table below.

- On page 3 (General / Inputs / Lock), tick “Create locked output”.

- Set Compensate clock to c2 (the 165 MHz 0° output).

- Click Finish. Add the generated altpll_core.qip to your project.

| Output | Frequency | Multiply | Divide | Phase shift | Usage |
| --- | --- | --- | --- | --- | --- |
| c0 | 24.000 MHz | 12 | 25 | 0° | OV7670 XCLK + camera_subsystem clock |
| c1 | 25.000 MHz | 1 | 2 | 0° | VGA pixel clock (vga_core, vga_interface) |
| c2 | 165.000 MHz | 33 | 10 | 0° | SDRAM controller FSM (internal) |
| c3 | 165.000 MHz | 33 | 10 | −90° (−1515 ps) | DRAM_CLK output pin — compensates PCB trace delay |

DRAM_CLK must be connected directly to the c3 PLL output pin. It must never pass through any FPGA logic or routing. This is already implemented correctly in stream_top.vhd: DRAM_CLK <= clk_dram.

# 5  Hardware Connections

## 5.1  Power supply wiring

The OV7670 requires three supply rails not available on the GPIO header. Build the following regulator circuit before connecting the camera.

| OV7670 pin | Voltage | Regulator | Input | Decoupling |
| --- | --- | --- | --- | --- |
| AVDD | 2.8V | AMS1117-2.8 | 5V from JP1 pin 29 | 100 nF + 10 µF at AVDD pin |
| DVDD | 2.8V | AMS1117-2.8 (shared) | Same regulator as AVDD | 100 nF + 10 µF at DVDD pin |
| DOVDD | 1.8V | AMS1117-1.8 | 5V from JP1 pin 29 | 100 nF + 10 µF at DOVDD pin |
| GND | 0V | — | DE10-Lite GND (JP1 pin 30) | Star point near camera module |

Power sequencing: apply DOVDD (1.8V) before AVDD/DVDD (2.8V). The firmware handles this automatically — i2c_top.vhd holds RESET_N low for 5 ms after the global reset releases, giving both supplies time to stabilise.

## 5.2  Level shifter wiring

All 14 signal lines between the OV7670 and the FPGA must pass through two TXS0108E level shifters (8 channels each).

- TXS0108E — chip A: connect VA to DOVDD (1.8V), VB to 3.3V, OE to 3.3V.

- TXS0108E — chip B: same power connections.

- Route D[7:0] through chip A (8 data lines).

- Route PCLK, HREF, VSYNC, XCLK, SIOC, SIOD through chip B (6 control/clock lines).

- RESET_N and PWDN can also go through chip B (2 remaining channels).

## 5.3  GPIO_0 header pin assignments

Connect the 3.3V side of the level shifters to the DE10-Lite GPIO_0 header (JP1) as follows. The 1.8V side connects to the OV7670 module.

| JP1 pin | GPIO_0 index | Direction | OV7670 | Signal | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | VCC3P3 | power | VCC | 3.3V | 100 nF decoupling cap at header pin |
| 2 | GND | power | GND | Ground |  |
| 3 | GPIO_0[0] | FPGA ← cam | D0 | D0 | Pixel LSB |
| 4 | GPIO_0[1] | FPGA ← cam | D1 | D1 |  |
| 5 | GPIO_0[2] | FPGA ← cam | D2 | D2 |  |
| 6 | GPIO_0[3] | FPGA ← cam | D3 | D3 |  |
| 7 | GPIO_0[4] | FPGA ← cam | D4 | D4 |  |
| 8 | GPIO_0[5] | FPGA ← cam | D5 | D5 |  |
| 9 | GPIO_0[6] | FPGA ← cam | D6 | D6 |  |
| 10 | GPIO_0[7] | FPGA ← cam | D7 | D7 MSB | Pixel MSB |
| 11 | GPIO_0[8] | FPGA ← cam | PCLK | PCLK | Pixel clock from camera (~24 MHz) |
| 12 | GPIO_0[9] | FPGA ← cam | HREF | HREF | Line valid, active high |
| 13 | GPIO_0[10] | FPGA ← cam | VSYNC | VSYNC | Frame sync, active high |
| 14 | GPIO_0[11] | FPGA → cam | XCLK | XCLK | 24 MHz master clock to camera from PLL c0 |
| 15 | GPIO_0[12] | bidirectional | SIOD | SIOD | SCCB data — 4.7 kΩ pull-up to 3.3V |
| 16 | GPIO_0[13] | FPGA → cam | SIOC | SIOC | SCCB clock — 4.7 kΩ pull-up to 3.3V |
| 17 | GPIO_0[14] | FPGA → cam | RESET_N | RESET_N | Active low hardware reset |
| 18 | GPIO_0[15] | FPGA → cam | PWDN | PWDN | Power-down, drive low (camera enabled) |

# 6  Building the Project in Quartus

## 6.1  Create the Quartus project

- Open Quartus Prime 18.1 Lite Edition.

- File → New Project Wizard. Set the project name to stream_top and the top-level entity to stream_top.

- Select device: MAX 10 family, device 10M50DAF484C7G.

- Add all 15 .vhd source files to the project.

- Skip the EDA tool settings for now; click Finish.

## 6.2  Generate and add the PLL IP

- Follow the steps in Section 4 to generate altpll_core.qip.

- In Quartus: Project → Add/Remove Files. Browse to altpll_core.qip and add it.

## 6.3  Apply pin assignments

- Open stream_top.qsf in a text editor and verify the GPIO_0 pin numbers match your physical wiring.

- In Quartus: Assignments → Import Assignments. Select stream_top.qsf.

The most common source of errors is a mismatch between the .qsf pin numbers and the actual wires. Cross-check every GPIO_0 entry against the DE10-Lite User Manual (Rev. C) before compiling.

## 6.4  Compile

- Processing → Start Compilation (Ctrl+L).

- Expected result: 0 errors. Warnings about unconstrained paths in the async FIFO crossing are expected and safe — the false_path constraints in stream_top.qsf suppress them.

- The fit should use approximately 8,000–10,000 LEs out of the 49,760 available on the MAX10.

## 6.5  Program the FPGA

- Connect the DE10-Lite to your PC via the USB Blaster cable.

- Tools → Programmer. Click Hardware Setup and confirm the USB Blaster is detected.

- Click Add File, navigate to output_files/stream_top.sof.

- Tick the Program/Configure checkbox and click Start.

- The green DONE LED on the DE10-Lite will light when programming succeeds.

# 7  First Power-On and Verification

## 7.1  Startup sequence

After programming, the following sequence happens automatically without any user interaction:

| Time | LED state | What is happening |
| --- | --- | --- |
| 0 ms | LEDR[0]=0, LEDR[1]=0 | PLL is locking. All outputs held in reset. |
| ~1 ms | LEDR[1]=1 | PLL locked. Global rst_n releases. SDRAM initialisation begins (200 µs wait + precharge + 2×refresh + MRS). |
| ~5 ms | LEDR[0]=0 | SDRAM init complete. Camera held in hardware reset (RESET_N low for 5 ms). |
| ~10 ms | LEDR[0]=0 | Camera reset released. SCCB register writes begin (~30 registers at 100 kHz). |
| ~30 ms | LEDR[2]=1 | SCCB configuration complete (i2c_done). Camera now in RGB565 640×480 mode. |
| ~33 ms | Image visible | First valid frame captured and displayed. Live video stream begins. |

## 7.2  LED status indicators

| LED | Signal | Meaning when lit |
| --- | --- | --- |
| LEDR[0] | pll_locked | All PLL clocks are stable and locked |
| LEDR[1] | rst_n | System is out of reset and running |
| LEDR[2] | i2c_done | OV7670 has been configured via SCCB (stays on permanently) |
| LEDR[3] | cam_fifo_empty | Camera FIFO is empty — normal during blanking, problem if always lit |
| LEDR[4] | vga_fifo_full | VGA FIFO is full — back-pressure from slow VGA drain, should be rare |
| LEDR[5] | frame_done | Pulses at 30 fps with each completed camera frame (too fast to see individually) |

## 7.3  Expected normal operation

- LEDR[0], [1], [2] all permanently lit within 30 ms of power-on.

- VGA monitor shows live 640×480 colour image from the camera.

- No tearing, rolling, or colour artefacts in stable lighting.

- Pressing KEY[0] resets the system and restarts the full startup sequence.

# 8  Clock Domain Architecture

The design uses three independent clock domains separated by asynchronous FIFOs with gray-code pointers. Understanding this is important for debugging and for any future modifications.

| Domain | Frequency | PLL output | Modules running in this domain |
| --- | --- | --- | --- |
| Camera | 24 MHz | c0 | i2c_top, camera_interface (write side of camera FIFO), OV7670 XCLK output |
| OV7670 PCLK | ~24 MHz | external | camera_interface (clocked by camera-generated PCLK, not PLL c0) |
| SDRAM | 165 MHz | c2 | sdram_controller, sdram_interface, both FIFO read/write bridge sides |
| DRAM_CLK | 165 MHz −90° | c3 | Goes directly to DRAM_CLK output pin. No FPGA logic. Phase-compensates PCB delay. |
| VGA | 25 MHz | c1 | vga_core, vga_interface (read side of VGA FIFO) |

## 8.1  Clock domain crossings

Three CDC boundaries exist, each protected by a dedicated asyn_fifo instance using gray-code pointer synchronisation:

- PCLK (24 MHz) → 165 MHz: camera pixel data, inside camera_subsystem. FIFO depth: 512 entries.

- 165 MHz → 25 MHz: display pixel data, inside vga_subsystem. FIFO depth: 1024 entries.

- PCLK → 165 MHz: frame_done pulse, re-synchronised inside sdram_subsystem using a 2-FF synchroniser.

The Quartus QSF contains set_false_path constraints for all three crossings. These are correct by design — do not remove them. Removing them will cause TimeQuest to flag false timing violations on the gray-code pointer paths.

# 9  OV7670 Camera Configuration

## 9.1  SCCB initialisation

The i2c_top.vhd module automatically writes all required registers on startup. No manual intervention is needed. The sequence is:

- Hold RESET_N low for 5 ms (hardware reset).

- Release RESET_N. Wait 5 ms for internal camera boot.

- Write COM7 = 0x80 (soft reset all registers).

- Wait 1 ms (mandatory post-soft-reset delay).

- Write remaining registers (clock, format, resolution, colour matrix).

- Assert i2c_done = 1. camera_interface begins capturing.

## 9.2  Key register settings

| Register | Address | Value | Purpose |
| --- | --- | --- | --- |
| COM7 | 0x12 | 0x04 | RGB output mode, no QVGA scaling |
| COM15 | 0x40 | 0xD0 | RGB565 format, full output range 0x00–0xFF |
| COM8 | 0x13 | 0xE0 | Disable AGC/AWB/AEC for stable colour in fixed lighting |
| COM10 | 0x15 | 0x00 | VSYNC active high, HREF no inversion (matches camera_interface.vhd) |
| CLKRC | 0x11 | 0x01 | Internal clock pre-scalar ÷2 → 12 MHz internal |
| DBLV | 0x6B | 0x4A | Bypass internal PLL, use direct clock |
| HSTART | 0x17 | 0x13 | Horizontal window start (640 px active) |
| VSTOP | 0x1A | 0x7A | Vertical window stop (480 lines active) |

To change camera settings (brightness, gain, white balance), modify the REG_TABLE constant in i2c_top.vhd and recompile. The table is an array of (address, data) pairs; add or modify entries as needed.

## 9.3  Pixel data format

The OV7670 outputs pixels as two bytes per PCLK cycle when HREF is high. The camera_interface.vhd assembles them into one 16-bit RGB565 word:

| Byte | D[7:0] content | Bits assigned to | Notes |
| --- | --- | --- | --- |
| Byte 0 (first) | [R4:R0][G5:G3] | pixel[15:8] | Latched into d_latch on first PCLK with HREF=1 |
| Byte 1 (second) | [G2:G0][B4:B0] | pixel[7:0] | Combined with d_latch; word written to FIFO |

The assembled 16-bit word pixel[15:0] has format: R[4:0] G[5:0] B[4:0]. The 4-bit VGA DAC outputs use the top 4 bits of each channel: VGA_R = pixel[15:12], VGA_G = pixel[10:7], VGA_B = pixel[4:1].

# 10  SDRAM Frame Buffer

## 10.1  Memory layout

The 64 MB IS42S16320F SDRAM holds a double-buffered frame buffer. Two complete frames are stored simultaneously so the camera can write into one bank while the VGA display reads from the other, eliminating screen tearing.

| Region | Start address | End address | Contents |
| --- | --- | --- | --- |
| Bank 0 (frame A) | 0x000000 | 0x095FFF | 307,200 RGB565 pixels (640×480) |
| Bank 1 (frame B) | 0x096000 | 0x12BFFF | 307,200 RGB565 pixels (640×480) |
| Unused | 0x12C000 | 0x3FFFFF | Remaining ~62 MB, not used by this project |

## 10.2  Double-buffer operation

- On startup: camera writes to bank 0, VGA reads from bank 1.

- Each time a camera frame completes (frame_done pulse): the bank selector swaps. Camera switches to the other bank; VGA begins reading from the freshly written bank.

- The VGA read pointer is never reset by a bank swap — it continues from wherever it was, avoiding mid-scan tears.

- The read pointer wraps to 0 when it reaches the end of a full frame (307,200 words).

## 10.3  SDRAM timing

The SDRAM controller operates at 165 MHz and meets all IS42S16320F –6 speed-grade timing requirements:

| Parameter | Required | Implemented | Notes |
| --- | --- | --- | --- |
| Power-up wait | ≥20 ms | 200 µs = 33,000 cycles | IS42S16320F spec: 200 µs minimum |
| tRCD (row to col delay) | ≥18 ns | 3 cycles = 18.2 ns | Minimum satisfied |
| tRP (precharge time) | ≥18 ns | 3 cycles = 18.2 ns | Minimum satisfied |
| tRFC (refresh cycle) | ≥66 ns | 11 cycles = 66.7 ns | Minimum satisfied |
| tWR (write recovery) | ≥2 cycles | 2 cycles | Applied before every PRECHARGE after WRITE |
| CAS latency | 3 | 3 cycles | Set in mode register (MRS) |
| Refresh interval | ≤7.8 µs | 7.5 µs = 1237 cycles | 8192 rows in 64 ms |

# 11  Troubleshooting

## 11.1  No image on monitor

| Symptom / check | Solution |
| --- | --- |
| LEDR[0] not lit after 2 s | PLL failed to lock. Check that clk_50m (PIN_P11) assignment is correct and the 50 MHz oscillator is present. |
| LEDR[2] never lights | SCCB configuration failed. Check SIOC and SIOD pull-up resistors (4.7 kΩ to 3.3V). Verify TXS0108E is powered correctly (VA=1.8V, VB=3.3V). |
| Monitor shows “No signal” | VGA signals not reaching monitor. Check VGA_HS and VGA_VS pin assignments. Verify the VGA cable is connected. Some monitors need a few seconds to detect the signal. |
| Monitor shows black screen only | Image pipeline not running. LEDR[3] (cam_fifo_empty) should flicker. If permanently lit, camera is not producing PCLK — check XCLK connection and DOVDD supply. |
| LEDR[1] not lit | System still in reset. KEY[0] may be stuck or debounce filtering a floating input. Check KEY[0] wiring. |

## 11.2  Image problems

| Symptom | Likely cause and solution |
| --- | --- |
| Wrong colours (green tint, inverted) | COM7 register not set correctly. Verify byte 4 in REG_TABLE is (0x12, 0x04) for RGB mode. Check COM15 = 0xD0 for RGB565. |
| Image rolls vertically | VSYNC polarity mismatch. COM10 register (0x15) controls polarity. Current setting (0x00) = VSYNC active high. Invert in camera_interface.vhd if needed. |
| Horizontal colour stripes | HREF alignment issue. byte_sel is getting out of sync. Check that HREF falling edge resets byte_sel in camera_interface.vhd (it should — this is implemented in the corrected version). |
| Image appears shifted left/right | Windowing registers (HSTART/HSTOP/HREF) need adjustment. Modify in REG_TABLE inside i2c_top.vhd. |
| Flickering or unstable colours | AVDD/DVDD power supply noise. Add more decoupling capacitance (try 47 µF bulk caps). Ensure ground plane is solid. |
| Tearing (image splits horizontally) | rd_ptr being reset on frame_done. Verify you are using the corrected sdram_interface.vhd where rd_ptr is NOT reset on bank swap. |

## 11.3  Compilation errors

- Error: “cannot open altpll_core.vhd” — Generate the PLL IP core first (Section 4). The altpll_core.qip file must be added to the project.

- Error: “clock domain crossing” warnings — These are expected for the async FIFO paths. They are suppressed by set_false_path in stream_top.qsf. Errors (not warnings) indicate a missing constraint.

- Error: “DRAM_CLK” timing failure — Ensure DRAM_CLK is connected to PLL c3 (−90°). Check that stream_top.vhd has DRAM_CLK <= clk_dram and clk_dram is pll_main port clk_dram (not clk_165m).

# 12  Running the VUnit Testbenches

The project includes 78 VUnit testbenches across 10 test files covering every module. Tests can be run with GHDL (free, open-source) or ModelSim.

## 12.1  Install VUnit and GHDL

pip install vunit-hdl

# Ubuntu/Debian:

sudo apt install ghdl

# macOS:

brew install ghdl

## 12.2  Test files

| Test file | Tests | What it covers |
| --- | --- | --- |
| tb_asyn_fifo.vhd | 19 | CDC FIFO: all three clock pairs (24→65, 165→25, 165→24 MHz), full/empty flags, data integrity |
| tb_i2c_top.vhd | 11 | SCCB bus waveforms, register order, soft-reset delay, locked output |
| tb_camera_interface.vhd | 11 | Pixel assembly, byte alignment, FIFO backpressure, frame_done timing |
| tb_camera_subsystem.vhd | 9 | End-to-end: i2c_done gating, CDC pixel integrity, bank swap |
| tb_vga_core.vhd | 9 | VESA timing: H/V counters, sync pulses, active pixel count, frame period |
| tb_vga_interface.vhd | 12 | RGB565 → DAC mapping, blanking, underrun, sync pipeline, active_d |
| tb_vga_subsystem.vhd | 7 | Integration: CDC FIFO + timing + DAC |
| tb_sdram_controller.vhd | 14 | Init sequence, 200 µs wait, write/read, refresh priority, tWR |
| tb_sdram_interface.vhd | 14 | Arbitration, FIFO pop timing, address counters, bank swap, tearing fix |
| tb_sdram_subsystem.vhd | 10 | Full integration: SDRAM init, pixel round-trip, bank swap, DQM |

## 12.3  Running tests

# Run all tests

python run.py



# Run only camera tests

python run_camera.py



# Run only SDRAM tests

python run_sdram.py



# Run one specific test with waveform viewer

python run.py --gui *tc_pixel_assembly*

The SDRAM controller tests that wait for initialisation take ~200 µs of simulation time (33,000 cycles at 165 MHz). GHDL runs these in 2–5 seconds each — this is normal. The run.py scripts set --stop-time=5ms automatically for these tests.

# 13  Module Port Reference

This section lists the ports of each top-level subsystem for reference when integrating or modifying the design.

## 13.1  stream_top (top-level entity)

entity stream_top is

  port (

    MAX10_CLK1_50  : in    std_logic;          -- 50 MHz oscillator

    KEY            : in    std_logic_vector(1 downto 0);  -- push buttons

    LEDR           : out   std_logic_vector(9 downto 0);  -- debug LEDs

    OV7670_PCLK    : in    std_logic;

    OV7670_HREF    : in    std_logic;

    OV7670_VSYNC   : in    std_logic;

    OV7670_D       : in    std_logic_vector(7 downto 0);

    OV7670_XCLK    : out   std_logic;

    OV7670_SIOC    : out   std_logic;

    OV7670_SIOD    : inout std_logic;

    OV7670_RESET_N : out   std_logic;

    OV7670_PWDN    : out   std_logic;

    DRAM_CLK/CKE/CS_N/RAS_N/CAS_N/WE_N : out std_logic;

    DRAM_BA        : out   std_logic_vector(1 downto 0);

    DRAM_ADDR      : out   std_logic_vector(12 downto 0);

    DRAM_UDQM/LDQM : out  std_logic;

    DRAM_DQ        : inout std_logic_vector(15 downto 0);

    VGA_R/G/B      : out   std_logic_vector(3 downto 0);

    VGA_HS/VS      : out   std_logic

  );

end entity stream_top;

## 13.2  pll_main

entity pll_main is

  port (

    clk_50m  : in  std_logic;   -- 50 MHz input

    clk_24m  : out std_logic;   -- c0: 24 MHz,   0 deg  (camera XCLK)

    clk_25m  : out std_logic;   -- c1: 25 MHz,   0 deg  (VGA pixel clock)

    clk_165m : out std_logic;   -- c2: 165 MHz,  0 deg  (SDRAM FSM)

    clk_dram : out std_logic;   -- c3: 165 MHz, -90 deg (DRAM_CLK pin)

    locked   : out std_logic

  );

end entity pll_main;

# 14  Pre-Flight Checklist

Use this checklist before first power-on to catch the most common mistakes.

## Hardware

- TXS0108E level shifter: VA = 1.8V (DOVDD), VB = 3.3V, OE = 3.3V

- AMS1117-2.8 LDO regulator present, output measured at 2.8V before connecting camera

- AMS1117-1.8 LDO regulator present, output measured at 1.8V before connecting camera

- 4.7 kΩ pull-up on SIOC connected to 3.3V

- 4.7 kΩ pull-up on SIOD connected to 3.3V

- 100 nF ceramic capacitor on each OV7670 supply pin (AVDD, DVDD, DOVDD)

- 10 µF bulk capacitor on each OV7670 supply rail

- All OV7670 GND pins connected to DE10-Lite GND

- VGA cable connected from DE10-Lite to monitor

## Quartus project

- altpll_core.qip generated and added to project

- PLL outputs verified: c0=24 MHz, c1=25 MHz, c2=165 MHz 0°, c3=165 MHz −90°

- PLL ‘compensate clock’ set to c2 (165 MHz 0°)

- PLL ‘locked output’ ticked

- stream_top.qsf imported as pin assignments

- GPIO_0 pin numbers verified against your physical wiring

- Project compiles with 0 errors

- .sof file programmed to FPGA via USB Blaster

## Power-on verification

- LEDR[0] lights within 1 ms (PLL locked)

- LEDR[1] lights within 1 ms (system out of reset)

- LEDR[2] lights within 30 ms (SCCB config complete)

- VGA monitor shows live image within 35 ms