OV7670 Camera — DE10-Lite FPGA Connection Schematic

640 × 480 VGA, RGB565, real-time streaming  •  MAX10 10M50DAF484C7G

# 1  Signal connection schematic

All 14 signal lines between the OV7670 and the DE10-Lite GPIO_0 header pass through a TXS0108E 8-channel bidirectional level shifter (1.8V side A, 3.3V side B). The SCCB bus (SIOC, SIOD) additionally requires 4.7 kΩ open-drain pull-up resistors to 3.3V.

Figure 1 — Signal schematic: OV7670 ↔ TXS0108E level shifter ↔ DE10-Lite GPIO_0

# 2  Power supply schematic

The OV7670 requires three separate supply rails. None of these are available directly from the DE10-Lite GPIO header — they must be generated by external LDO regulators.

Figure 2 — Power supply: LDO regulators generating 2.8V and 1.8V from the DE10-Lite 5V rail

# 3  Power supply requirements

| Pin | Voltage | Function | Source | Decoupling |
| --- | --- | --- | --- | --- |
| AVDD | 2.8V | Analog pixel array power | AMS1117-2.8 from 5V | 100 nF + 10 µF |
| DVDD | 2.8V | Digital core power | AMS1117-2.8 shared | 100 nF + 10 µF |
| DOVDD | 1.8V | Digital I/O power | AMS1117-1.8 from 5V | 100 nF + 10 µF |
| GND | 0V | All grounds tied together | DE10-Lite GND | Star point near module |

Power-up sequence: apply DOVDD (1.8V) before AVDD/DVDD (2.8V). Hold RESET_N low until both supplies are stable. The i2c_top.vhd firmware already enforces this by keeping RESET_N low for 5 ms after global rst_n releases.

# 4  Pin-by-pin wiring table

JP1 is the 40-pin GPIO_0 header on the DE10-Lite board. All signal lines must pass through the TXS0108E before reaching the camera module.

| JP1 | GPIO_0 index | Direction | OV7670 | Signal | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | VCC3P3 | power | VCC | 3.3V | 100 nF + 10 µF decoupling near module |
| 2 | GND | power | GND | Ground | Common ground |
| 3 | GPIO_0[0] | FPGA ← cam | D0 | D0 | Pixel bit 0 |
| 4 | GPIO_0[1] | FPGA ← cam | D1 | D1 | Pixel bit 1 |
| 5 | GPIO_0[2] | FPGA ← cam | D2 | D2 | Pixel bit 2 |
| 6 | GPIO_0[3] | FPGA ← cam | D3 | D3 | Pixel bit 3 |
| 7 | GPIO_0[4] | FPGA ← cam | D4 | D4 | Pixel bit 4 |
| 8 | GPIO_0[5] | FPGA ← cam | D5 | D5 | Pixel bit 5 |
| 9 | GPIO_0[6] | FPGA ← cam | D6 | D6 | Pixel bit 6 |
| 10 | GPIO_0[7] | FPGA ← cam | D7 | D7 MSB | Pixel bit 7 (most significant) |
| 11 | GPIO_0[8] | FPGA ← cam | PCLK | PCLK | Pixel clock from OV7670 (~24 MHz) |
| 12 | GPIO_0[9] | FPGA ← cam | HREF | HREF | Line valid (high = valid pixel) |
| 13 | GPIO_0[10] | FPGA ← cam | VSYNC | VSYNC | Frame sync (high = blanking) |
| 14 | GPIO_0[11] | FPGA → cam | XCLK | XCLK | Master clock to camera — 24 MHz from PLL c0 |
| 15 | GPIO_0[12] | bidirectional | SIOD | SIOD | SCCB data — 4.7 kΩ pull-up to 3.3V |
| 16 | GPIO_0[13] | FPGA → cam | SIOC | SIOC | SCCB clock — 4.7 kΩ pull-up to 3.3V |
| 17 | GPIO_0[14] | FPGA → cam | RESET_N | RESET_N | Hardware reset (active low) |
| 18 | GPIO_0[15] | FPGA → cam | PWDN | PWDN | Power-down (tie low / drive 0 to enable camera) |

# 5  Critical design notes

## 5.1  Level shifting — mandatory

The OV7670 digital I/O operates at 1.8V (DOVDD rail). The DE10-Lite GPIO_0 header is 3.3V. Connecting directly will present 3.3V to OV7670 inputs whose absolute maximum is 1.8V + 0.3V = 2.1V — this will permanently damage the sensor.

- Use a TXS0108E (Texas Instruments) — 8-channel auto-direction, 1.8V ↔ 3.3V.

- VA pin → 1.8V (DOVDD), VB pin → 3.3V, OE pin → 3.3V (always enabled).

- You need 2 × TXS0108E (or 1 × TXS0102 for SCCB + 1 × TXS0108E for 8 data+sync lines).

- Place the level shifter as close to the camera module as possible to minimise trace length on the 1.8V side.

## 5.2  SCCB pull-up resistors

- SIOC and SIOD are open-drain. Both lines require 4.7 kΩ pull-up resistors to 3.3V.

- Place resistors on the 3.3V side of the level shifter (between the TXS0108E B-side and the FPGA).

- Without pull-ups, the SCCB bus will not reach logic-high and no registers will be written.

## 5.3  XCLK — 24 MHz master clock

- XCLK is driven from pll_main output c0 (24 MHz, 0°) via GPIO_0[11].

- This is a standard FPGA output — no external buffer is needed.

- The OV7670 will generate its own PCLK output after SCCB configuration is complete.

- PCLK frequency = XCLK / 2 when CLKRC register is set to 0x01 (pre-scalar = /2).

## 5.4  Decoupling capacitors

- Place one 100 nF ceramic capacitor and one 10 µF electrolytic capacitor at each power pin (AVDD, DVDD, DOVDD) directly on the camera module PCB.

- Power supply noise is the most common cause of unstable colour and image flickering.

- Add a 100 nF cap on the 3.3V line at the GPIO_0 header connector.

## 5.5  FPGA QSF pin assignments

The following assignments correspond to the GPIO_0 header on the DE10-Lite. Verify against the DE10-Lite User Manual (Rev. C) before programming.

| Signal | GPIO_0 index | QSF assignment example |
| --- | --- | --- |
| D[7:0] | GPIO_0[7:0] | PIN_AG28..PIN_AH27 |
| PCLK | GPIO_0[8] | PIN_AH26 |
| HREF | GPIO_0[9] | PIN_AH24 |
| VSYNC | GPIO_0[10] | PIN_AH23 |
| XCLK out | GPIO_0[11] | PIN_AG23 |
| SIOD | GPIO_0[12] | PIN_AF23 |
| SIOC | GPIO_0[13] | PIN_AG24 |
| RESET_N | GPIO_0[14] | PIN_AF24 |
| PWDN | GPIO_0[15] | PIN_AH25 |

These pin numbers are based on the standard DE10-Lite GPIO_0 header mapping. Always cross-reference with the Physical Constraints File in your Quartus project and the board schematic.